SWaT Lab Demo

Pre-Run Checklist

51.508 Secure Cyber Physical Systems  ·  iTrust Lab, SUTD

PLC1: 192.168.1.10   |   PLC1B: 192.168.1.11   |   Library: pylogix

Instructions

Work through each section in order. Tick ☐ as you confirm each item. Do not proceed to the next section until all boxes in the current section are ticked. If any item fails, stop and resolve before continuing.

# A. On Arrival

| A1 — Physical Setup |  |  |
| --- | --- | --- |
| ☐ | Locate the HMI workstation The Ubuntu Linux machine directly connected to the SWaT OT network | ⬜ TODO |
| ☐ | Log in to HMI Use provided credentials — do not use your own account | ⬜ TODO |
| ☐ | Open a terminal Check Python is available: python3 --version | ⬜ TODO |
| ☐ | Navigate to project folder cd ~/swat-cps-project (or wherever files are copied) | ⬜ TODO |
| ☐ | Confirm ae_model.pt and adv_model.pt are present ls -lh ae_model.pt adv_model.pt — both should show file sizes | ⬜ TODO |
| A2 — Dependencies |  |  |
| ☐ | Check pylogix is installed python3 -c "import pylogix; print(pylogix.__version__)" | ⬜ TODO |
| ☐ | Check torch is installed python3 -c "import torch; print(torch.__version__)" | ⬜ TODO |
| ☐ | Install missing packages if needed pip install pylogix torch numpy pandas scikit-learn | ⬜ TODO |

# B. PLC Connectivity Check

Run this script in a terminal first. All tags must return a value (not None) before proceeding.

python3 -c "

from pylogix import PLC

with PLC() as plc:

plc.IPAddress = '192.168.1.10'

tags = ['HMI_LIT101.Pv', 'AI_FIT_101_FLOW',

'HMI_MV101.Cmd', 'HMI_MV101.Auto',

'HMI_P101.Auto', 'HMI_LIT101.Sim',

'HMI_LIT101.Sim_PV']

for t in tags:

r = plc.Read(t)

print(f'{t:30s} = {r.Value}  [{r.Status}]')

"

| ✓ | Tag | Expected Value | Actual Value |
| --- | --- | --- | --- |
| ☐ | HMI_LIT101.Pv Tank level in mm | 300–850 mm | _____ mm |
| ☐ | AI_FIT_101_FLOW Flow rate in L/s | 0–2.6 L/s | _____ L/s |
| ☐ | HMI_MV101.Cmd Valve command | 2 = OPEN (normal) | _____ |
| ☐ | HMI_MV101.Auto Valve auto mode | True (auto mode) | _____ |
| ☐ | HMI_P101.Auto Pump auto mode | True (running) | _____ |
| ☐ | HMI_LIT101.Sim Simulation mode | False (off) | _____ |
| ☐ | HMI_LIT101.Sim_PV ⚠ Check capitalisation — Sim_PV vs Sim_Pv | Success status | _____ |

⚠ If Sim_PV returns error, try Sim_Pv (lowercase v). Update phase2_spoof.py accordingly before running the attack.

# C. Tag Name Verification

Allen-Bradley tag names are case-sensitive. Verify each tag reads a real value before the demo.

| ✓ | Tag Name | Used In | Status |
| --- | --- | --- | --- |
| ☐ | HMI_LIT101.Pv Level sensor tag | invariant_checker, autoencoder_detector, phase2_spoof | ⬜ Confirmed |
| ☐ | AI_FIT_101_FLOW Flow sensor tag | invariant_checker, autoencoder_detector, phase2_spoof | ⬜ Confirmed |
| ☐ | HMI_MV101.Cmd Valve command | invariant_checker, phase1_inject, recovery_agent | ⬜ Confirmed |
| ☐ | HMI_MV101.Auto Valve auto mode | invariant_checker, phase1_inject | ⬜ Confirmed |
| ☐ | HMI_P101.Auto Pump auto mode | invariant_checker, phase1_inject, recovery_agent | ⬜ Confirmed |
| ☐ | HMI_LIT101.Sim Sensor sim enable | phase2_spoof | ⬜ Confirmed |
| ☐ | HMI_LIT101.Sim_PV Sensor sim value | phase2_spoof | ⬜ Check case! |

# D. Pre-Demo Setup

| D1 — Open 5 terminals |  |  |
| --- | --- | --- |
| ☐ | Terminal 1 Reserved for phase1_inject.py (attack) | ⬜ READY |
| ☐ | Terminal 2 Reserved for phase2_spoof.py (evasion) | ⬜ READY |
| ☐ | Terminal 3 Reserved for invariant_checker.py (Layer 1) | ⬜ READY |
| ☐ | Terminal 4 Reserved for autoencoder_detector.py (Layer 2) | ⬜ READY |
| ☐ | Terminal 5 Reserved for fusion.py (Layer 3 — auto-triggers recovery) | ⬜ READY |
| D2 — Start defense layer first |  |  |
| ☐ | Terminal 3: Start invariant checker python3 scripts/defense/invariant_checker.py --plc-ip 192.168.1.10 | ⬜ RUNNING |
| ☐ | Terminal 4: Start autoencoder detector python3 scripts/defense/autoencoder_detector.py monitor --plc-ip 192.168.1.10 --model ae_model.pt | ⬜ RUNNING |
| ☐ | Terminal 5: Start fusion engine python3 scripts/defense/fusion.py | ⬜ RUNNING |
| ☐ | Wait for all 3 to show normal readings Look for: inv_flag=0, MSE below threshold, "All clear" — wait at least 30 seconds | ⬜ CONFIRMED |
| D3 — Safety checks before attack |  |  |
| ☐ | Get lab engineer sign-off Do not run phase1_inject.py without explicit lab engineer approval | ⬜ SIGNED OFF |
| ☐ | Confirm TA is physically present Required for all experiments on the SWaT testbed | ⬜ PRESENT |
| ☐ | Confirm PLC1B is reachable for failover python3 -c "from pylogix import PLC; plc=PLC(); plc.IPAddress='192.168.1.11'; print(plc.Read('HMI_LIT101.Pv').Status)" | ⬜ REACHABLE |
| ☐ | Note current LIT101 level Record here: _______ mm. Attack should be run when level is in normal range (400–750mm) | ⬜ NOTED |

# E. Demo Sequence

| E1 — Run attack (after defense is running) |  |  |
| --- | --- | --- |
| ☐ | Terminal 1: Start Phase 1 injection python3 scripts/attack/phase1_inject.py --plc-ip 192.168.1.10 --duration 120 | ⬜ STARTED |
| ☐ | Terminal 2: Start Phase 2 spoofing python3 scripts/attack/phase2_spoof.py attack --plc-ip 192.168.1.10 --model adv_model.pt --duration 120 | ⬜ STARTED |
| E2 — Watch defense respond |  |  |
| ☐ | Terminal 4 (AE): Should NOT fire immediately Phase 2 is fooling the ML detector — MSE should stay below threshold. This demonstrates evasion. | ⬜ OBSERVE |
| ☐ | Terminal 3 (Invariant): Should fire within 6 seconds Look for: *** VIOLATION *** I-9: MV101=CLOSED and P101.Auto=False — physics layer catches it | ⬜ OBSERVE |
| ☐ | Terminal 5 (Fusion): Score ≥ 1.5 → ALERT After 3 consecutive flagged cycles, fusion auto-launches recovery_agent.py | ⬜ OBSERVE |
| E3 — Recovery |  |  |
| ☐ | Recovery auto-launches from Terminal 5 Should see: STEP 1 DETECT → STEP 2 CONTAIN → STEP 3 SAFE STATE → STEP 4 FAILOVER → STEP 5 VERIFY | ⬜ OBSERVE |
| ☐ | Verify 5 clean invariant cycles Recovery completes when 5 consecutive clean cycles confirmed. Target: <15 seconds total. | ⬜ CONFIRMED |
| ☐ | Record total recovery time Time from ALERT to *** RECOVERY COMPLETE ***: _______ seconds | ⬜ RECORDED |

# F. Troubleshooting

| Problem | Fix |
| --- | --- |
| Tag returns None or error | Run get_tag_list() to find exact tag names. Check capitalisation (Sim_PV vs Sim_Pv). |
| pylogix not found | pip install pylogix — then re-run script |
| Invariant checker not detecting attack | Check P101.Auto is actually changing to False when attack runs. Print state each cycle. |
| AE detector keeps firing false alarms | Threshold may be too low for the real plant. Retrain with: python3 scripts/defense/autoencoder_detector.py train --data assets/19-Feb-2026_0930_1735.csv --save ae_model.pt |
| Recovery fails to restore PLC | Manually restore via HMI: set MV101.Auto=True, P101.Auto=True. Tell the lab engineer immediately. |
| PLC1B not reachable for failover | Skip Step 4 in recovery demo. Mention in presentation that failover requires lab confirmation of PLC1B IP. |
| Phase 2 Sim tags not accepted | PLC may have Sim mode disabled. Demo Phase 1 only and explain Phase 2 conceptually from the adversarial model results. |

# G. Quick Reference

| PLC IPs | PLC1: 192.168.1.10 PLC2: 192.168.1.20 PLC3: 192.168.1.30 |
| --- | --- |
| Redundant PLCs | PLC1B: 192.168.1.11 PLC2B: 192.168.1.21 |
| Normal LIT101 range | 300–850 mm ¦ LL alarm: 250mm ¦ HH alarm: 800mm |
| MV101 Cmd values | 1 = CLOSE ¦ 2 = OPEN |
| P101 Cmd values | 1 = OFF ¦ 2 = ON |
| Fusion threshold | score = 1.5 × inv_flag + 1.0 × ae_flag ¦ ALERT if ≥ 1.5 for 3 cycles |
| AE threshold | MSE > 0.000950 → ae_flag = 1 (trained on 19-Feb-2026 dataset) |
| Recovery target | < 15 seconds from ALERT to verified safe state |
| Lab booking | https://itrustestbed.simplybook.asia/v2/ |

— End of Checklist —